KL UNIVERSITY, HYDERABAD

Department of Computer Science & Engineering

PROJECT BASED LEARNING (PBL)

DETAILED PROJECT REPORT

AgroSense

Smart Greenhouse Monitoring & Irrigation Management System

Submitted by

2520040096 – K. Prasanna
2520040118  –  A. Sai Pranay

Team No: 20

Under the guidance of

Dr. Karumuri Sri Rama Murthy

Assistant Professor, Department of CSE

KLU Hyderabad

Academic Year 2025 – 2026

CERTIFICATE

This is to certify that the project report entitled “AgroSense – Smart Greenhouse Monitoring & Irrigation Management System” is a bonafide work carried out by K. Prasanna (2520040096) and A. Sai Pranay (2520040118), Team No. 20, students of the Department of Computer Science and Engineering, KLU Hyderabad, as part of the Project Based Learning (PBL) course.

The work has been carried out under my supervision and guidance.

Dr. Karumuri Sri Rama Murthy

Assistant Professor, Department of CSE

KLU Hyderabad

Signature: ____________________          Date: ______________

DECLARATION

We, the members of Team No. 1, hereby declare that the project report entitled “AgroSense – Smart Greenhouse Monitoring & Irrigation Management System” submitted to the Department of Computer Science and Engineering, KLU Hyderabad, is an original work carried out by us under the guidance of Dr. Karumuri Sri Rama Murthy. The work has not been submitted to any other university or institution for the award of any degree or diploma.

K. Prananna (2520040096)                     ____________________

A. Sai Pranay (2520040118)                   ____________________

ACKNOWLEDGEMENT

We express our sincere gratitude to our project guide, Dr. Karumuri Sri Rama Murthy, Assistant Professor, Department of Computer Science and Engineering, KLU Hyderabad, for his valuable guidance, constant encouragement and suggestions throughout the development of this project.

We thank the Head of the Department and all faculty members of the Department of CSE for providing the facilities and a supportive environment. We also thank our friends and families for their continuous support.

Team No. 1: T. Sai Pavan Reddy, K. Kushal Kumar, A. Sai Pranay

ABSTRACT

Greenhouse farming depends on keeping soil moisture, temperature, humidity and light within the range required by each crop. Manual monitoring is slow, labour-intensive and error-prone, and fixed-schedule watering wastes water. AgroSense is a web-based smart greenhouse monitoring and irrigation management system that addresses these problems.

The system monitors three crop zones – Tomato (Zone A), Capsicum (Zone B) and Lettuce (Zone C). ESP32-based sensor nodes measure soil moisture, air temperature, humidity and light intensity and publish JSON telemetry over MQTT. A Node.js/Express server validates the readings, stores them in PostgreSQL/TimescaleDB (or in memory in demo mode), evaluates crop-specific threshold rules and raises warning or critical alerts. When automatic mode is enabled, the server switches the irrigation relay on and off using hysteresis thresholds; the grower can also override the pump manually. Live data are pushed to clients through Socket.IO and alerts through Server-Sent Events.

The user interface is a responsive React 18 dashboard with seven pages – Overview, Growing zones, Climate & trends, Irrigation, Alerts, Live sensors and Settings. A built-in simulator allows the whole system to be demonstrated without physical hardware. The project demonstrates a low-cost, extensible IoT architecture for precision agriculture.

Keywords: Smart agriculture, IoT, greenhouse, ESP32, MQTT, Node.js, React, TimescaleDB, automatic irrigation.

TABLE OF CONTENTS

1. Introduction	1

1.1 Background	1

1.2 Motivation	1

1.3 Objectives	1

1.4 Scope	1

2. Problem Statement	1

2.1 Problem Description	1

2.2 Existing System	1

2.3 Proposed System	1

3. System Requirements	1

3.1 Hardware Requirements	1

3.2 Software Requirements	1

3.3 Functional Requirements	1

3.4 Non-Functional Requirements	1

4. Technologies Used	1

4.1 Why these technologies?	1

5. System Design	1

5.1 Architecture	1

5.2 Data Flow	1

5.3 MQTT Topics	1

5.4 Database Design	1

5.5 REST API	1

6. Implementation	1

6.1 Project Structure	1

6.2 Firmware (ESP32 Node)	1

6.3 Backend Modules	1

6.4 Front-end Implementation	1

6.5 Deployment	1

6.6 Configuration	1

7. Output Screens	1

8. Testing	1

8.1 Test Approach	1

9. Results and Discussion	1

9.1 Current Limitations	1

10. Conclusion	1

11. Future Enhancements	1

12. References	1

(If the list above is empty, right-click it in Microsoft Word and choose “Update Field”.)

# 1. Introduction

## 1.1 Background

Agriculture is shifting from experience-based practice to data-driven precision farming. In protected cultivation such as greenhouses, crop yield and quality depend on microclimate parameters – soil moisture, air temperature, relative humidity and light intensity. Small deviations, if unnoticed for hours, can stress the plants and reduce yield. Internet of Things (IoT) technology makes it possible to sense these parameters continuously, transmit them to a server and act automatically.

## 1.2 Motivation

Small and medium growers often depend on manual inspection and fixed watering schedules. This leads to over-watering (wasted water and energy, root diseases), under-watering (wilting) and delayed reaction to heat. We were motivated to build an affordable, open-source system that gives a grower a single dashboard for all zones, warns early and can irrigate automatically.

## 1.3 Objectives

- To monitor soil moisture, temperature, humidity and light for multiple crop zones in near real time.

- To apply crop-specific moisture thresholds and generate warning/critical alerts.

- To automate irrigation using hysteresis control and also allow manual override.

- To store readings in a time-series database for trend analysis.

- To provide a responsive web dashboard for visualisation and control.

- To provide a built-in simulator so the system can be demonstrated without hardware.

## 1.4 Scope

The project covers the sensing firmware, MQTT communication, the Node.js backend with REST and real-time APIs, the PostgreSQL/TimescaleDB schema, deployment configuration, and the React web dashboard for three crop zones in a single greenhouse (“Greenhouse 01”). Multi-user authentication, machine-learning prediction and mobile applications are listed as future work.

# 2. Problem Statement

## 2.1 Problem Description

- Manual monitoring: growers check soil and climate by hand, so problems are noticed late.

- Water wastage: fixed-schedule watering over- or under-irrigates crops with different needs.

- No early warning: dry soil or high temperature is found only after damage has begun.

- No data history: without stored readings, trends cannot be analysed to improve yield.

- Different crop needs: tomato, capsicum and lettuce require different moisture thresholds.

## 2.2 Existing System

In the conventional approach, a worker inspects plants and soil visually or with a hand-held meter and waters on a timetable. Commercial greenhouse controllers exist but are expensive, closed and difficult for small farms to adopt or customise.

## 2.3 Proposed System

AgroSense continuously collects telemetry from every zone, compares it with the crop’s threshold, raises alerts, automatically controls the irrigation relay and stores history for analysis. All of this is presented on a single browser-based dashboard built entirely on open-source components.

Table 1: Comparison of existing and proposed systems

| Aspect | Existing (manual) system | AgroSense |
| --- | --- | --- |
| Monitoring | Periodic manual inspection | Continuous sensor readings |
| Irrigation | Fixed schedule, manual valve | Threshold-based automatic + manual override |
| Alerts | None | Warning / critical alerts per zone |
| Data storage | Paper notes, if any | PostgreSQL / TimescaleDB history |
| Crop-specific rules | Operator experience | Per-zone dry thresholds |
| Cost | Low effort tools or costly controllers | Low-cost ESP32 + open-source software |

# 3. System Requirements

## 3.1 Hardware Requirements

Table 2: Hardware components

| Component | Purpose |
| --- | --- |
| ESP32 development board | Wi-Fi enabled microcontroller; one node per zone |
| Soil-moisture probe (analog, GPIO 34) | Measures volumetric soil moisture; calibrated between dry = 3200 and wet = 1300 ADC counts |
| DHT22 sensor (GPIO 4) | Air temperature and relative humidity |
| LDR (GPIO 35) | Light intensity (scaled to ≈ 0–52,000 lux) |
| Relay module (GPIO 26) + water pump | Switches irrigation ON/OFF |
| Server / PC | Runs Node.js API, MQTT broker and database |

## 3.2 Software Requirements

Table 3: Software requirements

| Software | Version / Notes |
| --- | --- |
| Node.js | 18 or later (LTS) |
| npm packages (server) | express, cors, dotenv, mqtt, pg, socket.io |
| npm packages (client) | react 18, react-dom, recharts, socket.io-client, vite 5 |
| Database | PostgreSQL 16 with TimescaleDB extension (optional) |
| MQTT broker | Eclipse Mosquitto 2 |
| Firmware tools | Arduino IDE with WiFi, PubSubClient and DHT libraries |
| Browser | Any modern browser (Chrome, Edge, Firefox, Safari) |
| Operating system | Windows (one-click batch file provided), Linux, macOS |

## 3.3 Functional Requirements

- Display live soil moisture, temperature, humidity and light per zone.

- Show average values across zones on the overview page.

- Show zone status as Healthy, Needs water, Dry, Critical or Saturated.

- Generate, list and resolve alerts.

- Start/stop irrigation per zone and toggle automatic mode.

- Provide a sensor input form and sample-data refresh for demonstration.

- Retrieve historical readings by zone and time range.

## 3.4 Non-Functional Requirements

- Usability: responsive layout for desktop and mobile, clear navigation.

- Performance: readings every 2 s from nodes; dashboard refreshes every 3–5 s.

- Reliability: automatic fallback to in-memory storage and to the simulator when no database/broker is configured.

- Security: configurable CORS origins and API rate limiting (300 requests/min per IP).

- Portability: Docker Compose for broker/database, and Render/Vercel deployment descriptors.

# 4. Technologies Used

The project follows a three-part software stack – front-end, back-end/middleware and database – together with an IoT sensing layer.

Table 4: Technology stack

| Layer | Technology | Role in AgroSense |
| --- | --- | --- |
| Front-end | React 18, Vite 5, Recharts, Socket.IO client, CSS | Single-page dashboard, charts, controls |
| Back-end / middleware | Node.js, Express 4, Socket.IO, MQTT.js | REST API, real-time push, device bridge, rule engine |
| Middleware functions | CORS, express.json, request logger, rate limiter | Cross-origin access, parsing, logging, protection |
| Database | PostgreSQL, TimescaleDB (pg driver) | Persistent time-series storage of readings and alerts |
| Messaging | MQTT (Eclipse Mosquitto) | Lightweight publish/subscribe between nodes and server |
| IoT firmware | Arduino C++ on ESP32 (WiFi, PubSubClient, DHT) | Sensor reading and relay control |
| DevOps | Docker Compose, Render, Vercel | Local infrastructure and cloud deployment |

## 4.1 Why these technologies?

- React + Vite: component-based UI with fast development builds.

- Node.js + Express: event-driven runtime suited to many small, frequent sensor messages.

- MQTT: low-overhead protocol designed for constrained IoT devices; supports retained messages for relay commands.

- TimescaleDB: PostgreSQL extension that partitions data by time, making trend queries efficient.

- Socket.IO / SSE: push updates without polling.

# 5. System Design

## 5.1 Architecture

AgroSense uses a five-layer architecture. Sensor nodes in each zone publish telemetry to an MQTT broker; the Node.js backend subscribes, validates and stores the data, evaluates rules, and exposes REST, WebSocket and SSE interfaces; the React front end consumes these interfaces. Irrigation commands travel the opposite way as retained MQTT messages to the node’s relay.

Figure 1: System architecture of AgroSense

## 5.2 Data Flow

Each reading passes through a fixed pipeline: validation, persistence, broadcast, threshold evaluation and – if automatic mode is on – relay control.

Figure 2: Sensor data processing and auto-irrigation flow

## 5.3 MQTT Topics

Table 5: MQTT topics

| Topic | Direction | Payload |
| --- | --- | --- |
| agrosense/<zone>/telemetry | Node → Server | {"soil":58.2,"temp":26.4,"humidity":67,"light":18400} |
| agrosense/<zone>/irrigation | Server → Node | {"on":true} (retained) |

## 5.4 Database Design

Three tables are used. The readings table is converted into a TimescaleDB hypertable when the extension is available; otherwise plain PostgreSQL is used.

Table 6: Database tables

| Table | Columns | Notes |
| --- | --- | --- |
| zones | id (PK), name, crop, dry_threshold | Seeded with A–Tomato (32), B–Capsicum (30), C–Lettuce (38) |
| readings | time, zone (FK → zones.id), soil_moisture, temperature, humidity, light | Hypertable on time; index readings_zone_time (zone, time DESC) |
| alerts | id (PK, serial), time, zone, level, message | Written by the rule engine |

## 5.5 REST API

Table 7: REST and real-time interfaces

| Method | Endpoint | Description |
| --- | --- | --- |
| GET | /api/health | Returns status, uptime and store type (postgres or memory) |
| GET | /api/zones | All zones with relay state and latest reading |
| GET | /api/history?zone=&hours=&bucket= | Bucketed history; hours 1–168, default 6 |
| GET | /api/alerts | Latest 30 alerts |
| GET | /api/alerts/stream | Server-Sent Events stream of new alerts |
| POST | /api/irrigation/:zone | Body {"on":bool,"auto":bool}; sets pump state / auto mode |
| WS | Socket.IO events | telemetry and relay events pushed to clients |

# 6. Implementation

## 6.1 Project Structure

agrosense/

├── client/            React + Vite dashboard (App.jsx, components.jsx, styles.css)

├── server/            Node.js API

│   ├── src/           index.js, core.js, mqtt.js, simulator.js, store.js

│   └── db/schema.sql  PostgreSQL / TimescaleDB schema

├── firmware/          agrosense_node.ino (ESP32)

├── infra/             docker-compose.yml, mosquitto.conf

├── render.yaml        API deployment descriptor

└── RUN-AGROSENSE-WINDOWS.bat

## 6.2 Firmware (ESP32 Node)

Each node reads the soil probe, maps the raw ADC value between the calibration points to 0–100 %, reads temperature and humidity from the DHT22 and light from the LDR, and every 2 seconds publishes a JSON message to its telemetry topic. It subscribes to the irrigation topic and drives the relay pin HIGH or LOW according to the received command.

float soil = constrain(map(analogRead(SOIL_PIN), SOIL_DRY, SOIL_WET, 0, 100), 0, 100);

float light = analogRead(LDR_PIN) * 52000.0 / 4095.0;

snprintf(buf, sizeof(buf),

"{\"soil\":%.1f,\"temp\":%.1f,\"humidity\":%.1f,\"light\":%.0f}",

soil, dht.readTemperature(), dht.readHumidity(), light);

mqtt.publish(teleTopic.c_str(), buf);

## 6.3 Backend Modules

### 6.3.1 Core engine (core.js)

The ingest() function is the heart of the server. It validates that the zone exists and all four values are numbers, saves the reading, emits a telemetry event, evaluates alert rules and applies the automatic irrigation logic. Alerts for the same key are suppressed for 90 seconds to avoid flooding.

if (r.soil < z.dry * 0.8)  raise(z.id, 'critical', ...soil moisture is critically low...);

else if (r.soil < z.dry)    raise(z.id, 'warning',  ...soil is drying out...);

if (r.temp > 34)            raise(z.id, 'warning',  ...air temperature is high...);

const rl = relay[z.id];

if (rl.auto) {

if (!rl.on && r.soil < z.dry - 4) setRelay(z.id, true);   // pump ON

else if (rl.on && r.soil > 62)    setRelay(z.id, false);  // pump OFF

}

Table 8: Zone thresholds and derived trigger values

| Zone | Crop | Dry threshold | Critical alert below (80 %) | Pump ON below (dry − 4) |
| --- | --- | --- | --- | --- |
| A | Tomato | 32 % | 25.6 % | 28 % |
| B | Capsicum | 30 % | 24 % | 26 % |
| C | Lettuce | 38 % | 30.4 % | 34 % |

### 6.3.2 MQTT bridge (mqtt.js)

When MQTT_URL is configured, the server connects to the broker, subscribes to agrosense/+/telemetry, parses each payload and forwards it to ingest(). Relay commands emitted by the core are published to agrosense/<zone>/irrigation as retained messages so a node that reconnects immediately receives the latest command.

### 6.3.3 Simulator (simulator.js)

If no broker is configured, the simulator back-fills 24 hours of history at 5-minute intervals for each zone and then generates a new reading every 2 seconds. Soil moisture decreases slowly when the pump is off and rises when it is on, so automatic irrigation can be observed. Temperature, humidity and light follow a daily sinusoidal pattern.

### 6.3.4 Storage (store.js)

The store object hides the persistence mechanism. With DATABASE_URL set it uses a PostgreSQL pool and applies schema.sql at start-up; otherwise it keeps up to 20,000 readings in memory.

### 6.3.5 API server (index.js)

The Express application registers the logger, CORS, JSON parser and an IP-based rate limiter (300 requests per minute) before the routes. Socket.IO broadcasts telemetry and relay events, an SSE endpoint streams alerts, and if a built client exists in client/dist it is served by the same process.

## 6.4 Front-end Implementation

The dashboard is a React 18 single-page application styled with custom CSS. A sidebar lists seven pages; each navigation entry opens the page in a new browser tab (?page=<name>) so every part can be presented independently.

Table 9: Dashboard pages

| Page | Content |
| --- | --- |
| Overview | Greeting, live snapshot banner, four average metric cards (soil, temperature, humidity, light) and zone cards |
| Growing zones | Detailed card for each crop zone with status badge, readings and refresh control |
| Climate & trends | Average climate metrics and per-zone comparison bars |
| Irrigation | Per-zone start/stop button and automatic-mode switch |
| Alerts | List of alerts with resolve action; new alert when moisture crosses below the threshold |
| Live sensors | Automatic sensor monitor, live feed by zone and sensor input form |
| Settings | Greenhouse name and operator name |

In the current build the dashboard refreshes simulated readings every 3 seconds in the browser, so it works stand-alone for demonstration. The repository also contains reusable chart components (live climate line chart, soil-moisture area chart) and API/Socket.IO helpers for connecting the interface to the backend.

## 6.5 Deployment

- Windows: double-click RUN-AGROSENSE-WINDOWS.bat to start API and client, then open http://localhost:5173.

- Local infrastructure: docker compose up in /infra starts TimescaleDB (port 5432) and Mosquitto (port 1883).

- Cloud: render.yaml deploys the API (health check /api/health); vercel.json rewrites all routes to index.html for the client.

## 6.6 Configuration

Table 10: Environment variables (server/.env)

| Variable | Description |
| --- | --- |
| PORT | API port (default 4000) |
| CORS_ORIGIN | Comma-separated allowed web origins |
| DATABASE_URL | PostgreSQL connection string; empty = in-memory storage |
| PGSSL | Use SSL for the database connection |
| MQTT_URL, MQTT_USERNAME, MQTT_PASSWORD | Broker details; empty = built-in simulator |

# 7. Output Screens

The following screens were captured from the running AgroSense dashboard.

Figure 3: Overview dashboard

The overview page shows the average soil moisture, air temperature, humidity and light intensity across all zones, with zone cards listing each crop and its status.

Figure 4: Growing zones page

Each growing zone (Tomato – North greenhouse, Capsicum – East greenhouse, Lettuce – West greenhouse) is shown with its readings and a status badge such as Healthy or Needs water.

Figure 5: Climate & trends page

Figure 6: Irrigation control page

The irrigation page gives per-zone start/stop controls and an automatic-mode switch, along with the soil moisture and threshold for each zone.

Figure 7: Alerts & events page

Alerts appear when soil moisture drops below a zone’s threshold. Unresolved alerts are counted and can be marked resolved.

Figure 8: Live sensors page with automatic monitor

Figure 9: Workspace settings page

# 8. Testing

## 8.1 Test Approach

The project was tested at three levels: unit-level checks of the threshold rules, functional testing of the dashboard pages, and system-level checks of the data flow in simulator mode. The table lists the principal test cases and expected results; the “Observed” column is to be completed by the team during the review demonstration.

Table 11: Test cases

| ID | Test | Input | Expected result | Observed |
| --- | --- | --- | --- | --- |
| T1 | Healthy soil reading | Zone A soil = 50 % | No alert; status Healthy |  |
| T2 | Dry warning | Zone A soil = 30 % | Warning alert raised |  |
| T3 | Critical alert | Zone A soil = 20 % | Critical alert raised |  |
| T4 | High temperature | Temp = 36 °C | Heat warning raised |  |
| T5 | Auto pump ON | Auto on, soil = 27 % (Zone A) | Relay turned ON |  |
| T6 | Auto pump OFF | Pump on, soil = 63 % | Relay turned OFF |  |
| T7 | Manual override | POST /api/irrigation/A {"on":true} | Pump ON; auto mode off |  |
| T8 | Invalid payload | soil = "abc" | Reading ignored |  |
| T9 | Unknown zone | POST /api/irrigation/Z | HTTP 404 |  |
| T10 | Rate limit | > 300 requests / min | HTTP 429 |  |
| T11 | Alert de-duplication | Repeated low readings < 90 s | Single alert |  |
| T12 | Dashboard pages | Open all 7 pages | Each page opens and renders |  |
| T13 | Auto refresh | Wait 3 s | Values update automatically |  |
| T14 | Manual sensor input | Low soil value in form | Alert created in Alerts page |  |

# 9. Results and Discussion

The dashboard presents the state of all three zones at a glance, and the colour-coded status badges make it easy to identify the zone that needs attention. In simulator mode, the backend demonstrates the complete closed loop: soil moisture falls, an alert is raised, the pump is switched on at the trigger value, moisture rises and the pump is switched off at 62 %. Because history is stored per zone, trends can be retrieved for up to 168 hours.

## 9.1 Current Limitations

- The packaged web interface runs on simulated browser-side data; connecting it to the live API/Socket.IO feed is the next integration step.

- No user authentication or role management.

- Thresholds are fixed in code/database rather than editable from the UI.

- Hardware tests depend on sensor calibration for each soil type.

# 10. Conclusion

AgroSense demonstrates an end-to-end smart greenhouse solution: ESP32 sensing nodes, MQTT transport, a Node.js rule engine with REST and real-time interfaces, time-series storage and a responsive React dashboard. It reduces manual monitoring, provides early warnings, and automates irrigation with crop-specific thresholds, while remaining low-cost and built on open-source tools. The built-in simulator makes the system easy to demonstrate and test.

# 11. Future Enhancements

- Live integration: wire the React dashboard to the Socket.IO/REST backend and real ESP32 nodes.

- Predictive analytics: machine-learning models to forecast soil-moisture decline and recommend irrigation timing.

- Notifications: SMS, WhatsApp or e-mail alerts and a mobile application.

- User management: authentication, roles and multi-greenhouse support.

- More sensors/actuators: pH, EC, CO₂, fans, vents, shade nets and fertigation.

- Solar and LoRa nodes: for large farms without Wi-Fi coverage.

- Editable thresholds: per-crop settings stored in the database and editable from the UI.

# 12. References

- Node.js Documentation. https://nodejs.org/docs

- Express.js Documentation. https://expressjs.com

- React Documentation. https://react.dev

- Vite Documentation. https://vitejs.dev

- Recharts Documentation. https://recharts.org

- Socket.IO Documentation. https://socket.io/docs

- Eclipse Mosquitto MQTT Broker. https://mosquitto.org

- MQTT Version 3.1.1 Specification, OASIS Standard.

- PostgreSQL Documentation. https://www.postgresql.org/docs

- TimescaleDB Documentation. https://docs.timescale.com

- Espressif Systems, ESP32 Series Datasheet.

- Aosong, DHT22 (AM2302) Digital Temperature and Humidity Sensor Datasheet.